Design1: Unoptimized Antenna

clc; clear; close all;

f = 15e9;

c = 3e8;

epsilon_r = 4.4;

h = 1.6e-3;

W = 6.1e-3;

L = 3.97e-3;

eff_epsilon = 3.51;

BW_required = 1.67e9;

Gain_required = 13.7;

lambda = c / f;

patch = patchMicrostrip;

patch.Length = L;

patch.Width = W;

patch.Height = h;

patch.Substrate = dielectric('FR4');

patch.GroundPlaneLength = L + 2*h;

patch.GroundPlaneWidth = W + 2*h;

patch.FeedOffset = [-L/3, 0];

figure;

show(patch);

title('Microstrip Patch Antenna Geometry');

freqRange = linspace(14e9, 16e9, 50);

s11 = sparameters(patch, freqRange);

figure;

rfplot(s11);

title('S11 Parameter (Return Loss)');

xlabel('Frequency (Hz)');

ylabel('S11 (dB)');

grid on;

s11_vals = rfparam(s11, 1, 1);

bw_mask = s11_vals < -10;  bw_freqs = freqRange(bw_mask);

if isempty(bw_freqs)

BW_simulated = 0;

else

BW_simulated = bw_freqs(end) - bw_freqs(1);

end

fprintf('Simulated Bandwidth: %.2f GHz\n', BW_simulated / 1e9);

figure;

pattern(patch, f);

title('Radiation Pattern of the Antenna');

G_simulated = pattern(patch, f, 0, 90);

fprintf('Simulated Gain: %.2f dBi\n', G_simulated);

figure;

pattern(patch, f, 'Type', 'directivity');

title('Directivity Pattern');

if abs(BW_simulated - BW_required) < 0.1e9 && abs(G_simulated - Gain_required) < 0.5

disp('Antenna design meets theoretical expectations.');

else

disp('Optimization needed to match theoretical values.');

end

Design2: Optimized Antenna

clc; clear; close all;

f = 15e9;

c = 3e8;

epsilon_r = 4.4;

h = 1.6e-3;

eff_epsilon = 3.51;

BW_required = 2e9;

Gain_required = 13.7;

%% Calculate Wavelength

wavelength = c / f;

%% Optimized Patch Dimensions

W = c / (2*f) * sqrt(2 / (epsilon_r + 1));

L = c / (2*f*sqrt(eff_epsilon)) - 2*h;

patch = patchMicrostrip;

patch.Length = L;

patch.Width = W;

patch.Height = h;

patch.Substrate = dielectric('FR4');

patch.GroundPlaneLength = L + 3*h;

patch.GroundPlaneWidth = W + 3*h;

patch.FeedOffset = [-L/3, 0];

figure;

show(patch);

title('Optimized Microstrip Patch Antenna');

freqRange = linspace(13.5e9, 16.5e9, 30);

s11 = sparameters(patch, freqRange);

figure;

rfplot(s11);

title('S11 Parameter (Return Loss)');

s11_vals = rfparam(s11, 1, 1);

bw_mask = s11_vals < -10;

bw_freqs = freqRange(bw_mask);

if isempty(bw_freqs)

BW_simulated = 0;

else

BW_simulated = bw_freqs(end) - bw_freqs(1);

end

fprintf('Simulated Bandwidth: %.2f GHz\n', BW_simulated / 1e9);

figure;

pattern(patch, f);

title('Radiation Pattern');

G_simulated = pattern(patch, f, 0, 90);

fprintf('Simulated Gain: %.2f dBi\n', G_simulated);

mesh(patch, 'MaxEdgeLength', wavelength/10);

if abs(BW_simulated - BW_required) < 0.1e9 && abs(G_simulated - Gain_required) < 0.5

disp('Optimized Antenna meets design goals.');

else

disp('Further optimization needed.');

end